BAHRIA UNIVERSITY, Karachi Campus

Department of Software Engineering

# REPORT

Course Title: Computer Communication & Networks Lab Course Code: CEL-223

Course Instructor: Dr. Hussain	Class: BSE-5(C)

Lab Instructor: Humaira Mehwish                                         Name: 	Fiza Khan

PROJECT TITLE:

Voice Over IP - Telephone Service Network

GROUP MEMBERS LIST:

| S.NO | Enrollment | Name |
| --- | --- | --- |
| 01 | 02-131232-052 | M. Abubakar Afzal |
| 02 | 02-131232-048 | Fiza Khan |
| 03 | 02-131232-118 | Alishba Maqsood |

SUBMISSION DATE: 12-12-2025

Contents

- INTRODUCTION	4

  - Introduction	4

  - Problem Statement	4

  - Proposed Solution	5

- DESIGN	6

  - Workflow Diagram	6

  - User Interfaces (Packet tracer, Physical View)	7

  - Network Diagram (Packet tracer, Logical View)	8

- METHODOLOGY	11

  - Technologies Used	11

    - Networking Devices	11

    - Protocols	11

    - Configuration Tools:	12

    - Additional Concepts:	13

  - Network Communication Methodology	14

  - Commands	16

- CONCLUSIONS AND FURTHER WORK	23

- REFERENCES	24

ABSTRACT

This project exhibits planning, design, and implementation of a comprehensive network infrastructure enabled with VoIP to meet the needs of a multi-department organization. The primary objective for this network is to serve as a unified communication platform integrating voice and data traffic by utilizing Voice over IP (VoIP) for efficient and cost- effective telephony services.

The network architecture is designed to be scalable, available, and secure, thus making it possible to adapt to future growth and changes in organizational requirements without major reconfigurations. The system utilizes advanced features such as VLAN segmentation to separate voice and data traffic, thereby enhancing network performance and security. Dynamic IP addressing through DHCP servers is used to streamline network management, thereby reducing administrative overhead and making rapid device deployment possible.

# INTRODUCTION

### Introduction

The project focuses on designing and implementing a robust and scalable network infrastructure that seamlessly integrates data and voice communication for a multi- departmental organization. This infrastructure leverages cutting-edge networking technologies, including Voice over IP (VoIP), to modernize telephony systems and provide a unified communication platform.

VoIP technology enables voice communication to be transmitted over IP networks, reducing telecommunication costs and simplifying infrastructure management. The project ensures efficient and secure data flow alongside voice communication by employing VLAN segmentation, which segregates traffic to optimize performance and enhance security.

Using industry-standard Cisco devices such as routers and switches ,the network is designed to support multiple departments, each with its own dedicated VLANs for voice and data. This setup enables logical isolation of traffic while maintaining connectivity between departments through inter-VLAN routing.

In addition to supporting day-to-day operations, the network incorporates scalability and availability to accommodate future growth and evolving organizational needs. Advanced configurations, including dynamic IP allocation using DHCP, secure remote access via SSH, and routing through the OSPF protocol, ensure that the infrastructure is efficient, secure, and adaptable.

This project demonstrates a real-world application of networking concepts, creating a foundation for reliable communication and data exchange across the organization. It is designed to meet modern business demands by integrating key features such as secure voice and data communication, centralized management, and easy scalability.

### Problem Statement

Modern organizations frequently encounter significant challenges in managing distinct networks for voice and data communication. Maintaining separate infrastructures not only increases operational costs but also introduces inefficiencies in resource utilization,

administration, and scalability. This fragmentation often results in communication bottlenecks, inconsistent service quality, and heightened complexity in troubleshooting.

Furthermore, ensuring secure and reliable communication between departments is a critical requirement that traditional setups may fail to address adequately. Organizations also need network solutions that are scalable to handle future growth and evolving requirements while maintaining high availability and robust performance.

These challenges are compounded by the need for centralized management, dynamic resource allocation, and seamless integration of voice and data services, leaving many organizations with outdated and inflexible network infrastructures that hinder productivity and collaboration.

### Proposed Solution

To address these challenges, this project introduces a unified network infrastructure that integrates voice and data communication using Voice over IP (VoIP) technology. This approach eliminates the need for separate telephony systems and leverages existing IP networks to reduce costs and streamline operations.

Key features of the proposed solution include:

VLAN Segmentation: Logical separation of voice and data traffic to enhance network security and optimize performance.

Dynamic IP Allocation: Using DHCP servers to simplify network management and ensure efficient utilization of IP address space for both voice and data devices.

Advanced Routing: Implementation of the OSPF (Open Shortest Path First) protocol for dynamic and efficient routing across the network, ensuring reliability and adaptability to changing traffic patterns.

Secure Communication: Utilizing SSH for remote access to network devices, ensuring secure administration and management.

Scalability and Availability: A modular network design that accommodates future expansion and maintains service availability without disrupting operations.

# DESIGN

### Workflow Diagram

### User Interfaces (Packet tracer, Physical View)

### Network Diagram (Packet tracer, Logical View)

Logical Network:

Finance Department: 	Sales Department:

Server Room:

HR Department:

Routers:

# METHODOLOGY

### Technologies Used

Following are the technologies used:

#### Networking Devices:

| Category | Items | Quantity | Details |
| --- | --- | --- | --- |
| Routers | Cisco Routers | 4 | Inter-department routing |
| Switches | Cisco Switch | 4 | VLAN implementation |
| IP Phones | Cisco Phone | 40 | Voice communication |
| PCs | Generic PC | 40 | End-user data devices |

#### Protocols:

| Protocol | Purpose | Details |
| --- | --- | --- |
| DHCP | Dynamic IP Allocation | Automatically assigns IP addresses to devices in the network, including PCs in the data VLAN and IP phones in the voice VLAN. This eliminates the need for manual configuration and ensures efficient use of IP address space. The routers handle DHCP for voice, while a central DHCP server handles data. |
| OSPF | Dynamic Routing | Ensures efficient route calculation between routers by dynamically learning and propagating routes across the network. OSPF ensures optimal paths are selected for communication between |

|  |  | departments and is resilient to network topology changes. |
| --- | --- | --- |
| SSH | Secure Management | Provides encrypted remote access to routers and switches for secure configuration and management. This ensures that administrators can safely access devices to make changes or troubleshoot issues without exposing sensitive data to the network. |

#### Configuration Tools:

| Tool | Purpose | Details |
| --- | --- | --- |
| Cisco IOS | Device Configuration | Command-line interface (CLI) is used to configure networking devices such as routers and switches. Tasks include setting up VLANs, DHCP services, OSPF routing, SSH, and VoIP services to ensure seamless operation of the network |
| Packet Tracer | Network Simulation | Provides a virtual simulation environment for designing and testing the network topology. Packet Tracer is used to verify configurations, simulate VoIP calls, and troubleshoot connectivity issues before actual implementation. |

#### Additional Concepts:

| Concept | Purpose | Details |
| --- | --- | --- |
| VLANs | Traffic Segmentation | VLANs (Virtual Local Area Networks) logically divide the network into segments. Each department is assigned two VLANs (one for voice and one for data) to segregate traffic. VLAN 100 is used for voice, ensuring all IP phones are on the same logical network for easier management and call routing. |
| Router-on-a-stick | Inter-VLAN Routing | A single router interface is configured with multiple subinterfaces, each associated with a specific VLAN. This setup enables the router to handle traffic routing between VLANs, allowing seamless communication between devices in different VLANs while maintaining isolation. |
| VoIP | Telephony Services | Converts voice signals into digital packets for transmission over the IP network. VoIP eliminates the need for traditional telephony infrastructure by using existing network resources. Each phone is assigned a unique number and mapped to its dynamically assigned IP address, enabling efficient call routing and management. |

### Network Communication Methodology

The network is designed to facilitate both inter-department and intra-department communication efficiently, ensuring seamless operation for voice and data services. Below is a detailed breakdown of how communication occurs within the network:

- VLAN-Based Segmentation

  - Voice VLAN (172.16.100.0/24):

All IP phones in the network are assigned to VLAN 100, dedicated exclusively to voice traffic. This ensures that voice communication remains isolated from data traffic, improving call quality and reducing latency. The routers for each department dynamically assign IP addresses to the IP phones via their built-in DHCP services.

  - Data VLAN (192.168.100.0/24):

All PCs and data devices are assigned to separate VLANs based on departmental needs. The DHCP server located in the server room assigns dynamic IP addresses to these devices.

  - Traffic Isolation:

The use of VLANs ensures that traffic for voice and data remains logically separated, preventing interference and optimizing network performance. This also simplifies troubleshooting and enhances security.

- Inter-VLAN Routing

Routing between VLANs is achieved using the router-on-a-stick configuration:

  - Each department's router has sub-interfaces configured for both voice (VLAN 100) and data VLANs.

  - The router tags incoming traffic with VLAN IDs and routes the packets accordingly.

  - This setup allows PCs and IP phones in one VLAN to communicate with devices in another VLAN.

For example:

  - A PC in the Finance department (192.168.100.10) can access a server in the server room (192.168.100.5).

  - An IP phone in the HR department (172.16.100.203) can call an IP phone in the Sales department (172.16.100.302).

- VoIP Communication Mechanism

  - Dial Plan and Number Assignment:

Each department is assigned a unique dialing range (e.g., Finance uses 1XX, HR uses 2XX). The routers are configured with dial-peering to map phone numbers to specific IP addresses dynamically.

  - Voice Packet Transmission:

When a user dials a number, the router:

    - Matches the number to a predefined pattern in the dial plan.

    - Resolves the number to the IP address of the destination IP phone using the VoIP configuration.

    - Routes the call packets over the voice VLAN using IP routing.

  - Quality of Service (QoS):

The network prioritizes voice packets over data packets to ensure that calls have minimal delay, jitter, and packet loss, maintaining high call quality even during heavy data traffic.

- Secure Communication and Administration

  - Secure Remote Access (SSH):

All routers are configured with SSH to allow secure administrative access for configuration and troubleshooting. This ensures that sensitive network operations are protected against unauthorized access.

  - Encryption and Authentication:

Passwords on devices are encrypted, and banners warn unauthorized users. This enhances the security posture of the network.

- Dynamic IP Address Allocation

  - Voice IPs:

Routers for each department act as DHCP servers for the voice VLAN, dynamically assigning IPs (e.g., 172.16.100.101–172.16.100.254) to the IP phones. This eliminates the need for manual IP configuration and simplifies management.

  - Data IPs:

The central DHCP server in the server room assigns IPs in the data VLAN (e.g., 192.168.100.101–192.168.100.254) to PCs dynamically. This ensures scalability and ease of device integration.

- Communication Examples

  - Intra-Department Communication:

An HR IP phone calls another HR IP phone .The router matches the dialed number, identifies the IP address, and routes the call within the same VLAN.

  - Inter-Department Communication:

A Finance IP phone calls an HR IP phone .The router uses the dial plan to resolve the HR phone's number to its IP address and routes the packets over the network.

  - Data Communication:

A PC in the Sales department (192.168.100.110) accesses a file server in the server room (192.168.100.5). The request is routed via the router’s sub-interface for the data VLAN.

### Commands

Switch Configuration:

en config t

hostname FIN-Router enable password cipher line console 0 password cipher

login exit

banner motd #NO AUTHORIZED ACCESS, THIS IS AGAINST THE LAW#

service password-encryption no ip domain lookup

do wr

Explanation:

This commands configures a Cisco router by setting its hostname, enabling encrypted passwords for both privileged EXEC mode and console access, and securing console access with login prompts. It also adds a legal message banner, disables DNS lookups to prevent delays, and ensures all passwords are encrypted.

Trunking Configuration:

vlan 50 name DATA int fa 0/1

switchport mode trunk exit

int range fa 0/2-5 switchport mode access switchport access vlan 50 do wr

Explanation:

VLAN 50 is created and named "DATA." Port fa 0/1 is configured as a trunk port to carry traffic from multiple VLANs, while ports fa 0/2 to 0/5 are set as access ports assigned to VLAN 50. The configuration is saved to memory to persist after a reboot.

OSPF Configuration:

router ospf 10

network 10.10.10.8 0.0.0.3 area 0

network 10.10.10.12 0.0.0.3 area 0

network 192.168.100.64 0.0.0.31 area 0

network 172.16.100.64 0.0.0.31 area 0 exit

do wr

Explanation:

The router is configured to run OSPF process 10. Four network statements are added to include specific IP address ranges in OSPF Area 0, enabling OSPF routing for those networks. The configuration is saved to memory to ensure it persists after a reboot.

Dial Peering:

dial-peer voice 3 voip destination-pattern 2.. session target ipv4:10.10.10.2 exit

dial-peer voice 3 voip destination-pattern 4.. session target ipv4:10.10.10.5 exit

Explanation:

Two dial peers are configured for VoIP communication. The first dial-peer (voice 3) matches calls with a destination pattern of 2.. and forwards them to the IP address 10.10.10.2. The second dial-peer matches calls with a destination pattern of 4.. and routes them to 10.10.10.5. These dial peers are used to route voice traffic based on the destination pattern, enabling communication between different endpoints.

DHCP Configuration For Data:

int fa0/0.40 encapsulation dot1q 40

ip add 192.168.100.97 255.255.255.224

ip helper-address 192.168.100.130 exit

int fa0/0.100 encapsulation dot1q 100

ip add 172.16.100.97 255.255.255.224

ex

do wr

int fa0/1.50 encapsulation dot1q 50

ip add 192.168.100.129 255.255.255.248

exit do wr

Explanation:

Three subinterfaces are configured with 802.1Q encapsulation for VLANs 40, 100, and 50. Each subinterface is assigned an IP address and subnet mask, with IP helper-address configured on VLAN 40 for DHCP forwarding. The configurations are saved to memory.

DHCP Configuration For Voice:

ip dhcp excluded-address 172.16.100.33 ip dhcp pool HRVOICE

network 172.16.100.32 255.255.255.224

default-router 172.16.100.33

option 150 ip 172.16.100.33 exit

do wr

Explanation:

The command excludes the IP address 172.16.100.33 from the DHCP pool. A DHCP pool named HRVOICE is created for the network 172.16.100.32/27, with the default gateway set to 172.16.100.33 and option 150 configured to point to the TFTP server 172.16.100.33. The configuration is saved to memory

Telephone Service Configuration:

telephony-service max-dn 20

max-ephones 20

ip source-address 172.16.100.33 port 2000

auto assign 1 to 20 exit

ephone-dn 1

number 201

ephone-dn 2

number 202

ephone-dn 3

number 203

ephone-dn 4

number 204

ephone-dn 5

number 205

ephone-dn 6

number 206

ephone-dn 7

number 207

ephone-dn 8

number 208

ephone-dn 9

number 209

ephone-dn 10

number 210

Explanation:

The telephony service is configured with a maximum of 20 phones and directory numbers, using IP address 172.16.100.33 on port 2000. 10 ephone-dn entries are created, assigning phone numbers 201 to 210.

## CONCLUSIONS AND FURTHER WORK

This project successfully demonstrates the design, implementation, and deployment of a robust and scalable network that integrates both voice and data communication. By leveraging key networking technologies such as VLANs (Virtual Local Area Networks), DHCP (Dynamic Host Configuration Protocol), OSPF (Open Shortest Path First), and VoIP (Voice over Internet Protocol), the project has achieved a secure, efficient, and highly manageable infrastructure that supports the seamless transmission of voice and data across the network.

Key Findings:

- VLAN Implementation: The network's segmentation through VLANs provides enhanced security and better management of network resources by isolating traffic types. This is particularly important in a VoIP environment where prioritization of voice traffic ensures high-quality, uninterrupted calls, even in congested networks.

- Dynamic IP Addressing (DHCP): The use of DHCP simplifies network administration by automating the assignment of IP addresses, which reduces manual errors and ensures efficient allocation of addresses across the entire network.

- OSPF Routing Protocol: The integration of OSPF contributes to a highly dynamic routing environment, ensuring that the network adapts to changes in topology and quickly routes traffic along the most efficient paths, minimizing latency and maximizing throughput. This is critical for maintaining the quality of VoIP calls, where low latency and reliable routing are paramount.

- VoIP Integration: The deployment of VoIP allows for the integration of voice communication over the existing data network infrastructure, reducing the need for separate voice circuits and enabling cost-effective scalability. The quality of service (QoS) mechanisms implemented ensure that voice traffic is prioritized over non-time- sensitive data, thus preventing call degradation.

Challenges and Solutions:

Throughout the deployment, several challenges were encountered and addressed:

- Network Congestion: VoIP traffic is sensitive to network congestion, which can lead to issues like jitter, latency, and packet loss. This was mitigated by configuring QoS to prioritize voice packets, ensuring that voice calls maintained high quality even during periods of heavy data usage.

- Security Considerations: Voice traffic, like any other network traffic, is vulnerable to various security threats. The use of encryption protocols, firewalls, and secure VLANs has ensured that the communication is protected from eavesdropping and unauthorized access.

Further Work In Future:

To continue enhancing the infrastructure, several areas for further development have been identified:

- Scalability: As the network grows, the current setup could be scaled to accommodate more users, additional voice channels, and more complex routing. Future work will focus on optimizing network performance and load balancing as the demand for voice and data services increases.

- Wireless Connectivity: The integration of wireless connectivity into the network would enable mobility for users while still maintaining the integrity of the VoIP service. This could involve adding wireless access points (APs) and ensuring that voice traffic is prioritized over wireless networks, which might introduce challenges such as signal interference and bandwidth limitations.

- Fault Tolerance and Redundancy: To improve the reliability and uptime of the network, further work can focus on implementing more advanced fault tolerance mechanisms. This could include setting up additional redundant paths using protocols like HSRP (Hot Standby Router Protocol) or VRRP (Virtual Router Redundancy Protocol), as well as ensuring backup power systems are in place to prevent service interruptions.

- Advanced Telephony Features: Future development can include integrating more advanced telephony features such as video calling, voicemail, call forwarding, and automatic call distribution (ACD). This would not only improve the functionality of the VoIP system but also provide users with a richer communication experience.

- Interoperability with Other Systems: As the system evolves, integrating with other telecommunication systems or third-party services like cloud-based conferencing platforms or contact center solutions could provide added flexibility and capabilities.

## REFERENCES

Cisco Networking Academy: www.netacad.com

Cisco Configuration Examples: What is Cisco Packet Tracer? | Free Training and Download

Subnetting Practice: www.subnettingpractice.com